Low Pass Filter

- Low pass filters block high frequency content of the image

- High frequency content correspond to boundaries of the object.

Noise Removal

- To remove speckles/dots on an image

- Dots can be modeled as impulses or continuously varying (Gaussian noise)

- Can be removed by taking mean or median values of neighboring pixels (e.g 3x3 window)

- Equivalent to low pass filtering

Problem with low pass filtering

- May blur edges

- More advanced techniques: adaptive, edge preserving

Mean/Averaging Filter

Median Filter

- Median filter replaces the pixel at the center of the filter with the median value of the pixels falling beneath the mask.

- Median filter does not blur the image but it rounds the corners

Matlab Implementation

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

%% Section 1

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

% Print on Matlab

figure(Name="Section 1");

myimage = imread('cameraman.tif');

imshow(myimage)

% Convert Image into Double for processing on matlab

id_double = (myimage/255);

% Assign image height and width to r and c variables

[r,c] = size(id_double);

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

%% Section 2 :

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

% Array with ones

f = ones(3,3)/9;

% filter2 :

% Y = filter2(H,X) applies a finite impulse response filter to a matrix of

% data X according to coefficients in a matrix H.

% Y = filter2(H,X,shape) returns a subsection of the filtered data according

% to shape. For example, Y = filter2(H,X,'valid') returns only filtered

% data computed without zero-padded edges.

figure(Name="Section 2");

i1 = filter2(f,myimage,'same');

imshow(i1/255);

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

%% Section 3 :

% xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

% Same result via using fspecial function

figure(Name="Section 3");

f1= fspecial('average',[3,3]);

i2 = filter2(f1,myimage,'same');

imshow(i2/255);