Point Transformations using MATLAB

Original Image:

% Section 1

myimage = imread('cameraman.tif');

imshow(myimage)

Thresholding : Select Pixels with given values to produce binary image

%% Section 2

% Thresholding

i1 = myimage>120;

figure(Name="Section 2");

imshow(i1)

Increase the Intensity :

S = r + C  where C is constant and r is pixel value of the image and S is the output image.

%% Section 3

% Increase the intensity of the pixel

id = double(myimage); % Have to convert double before doing math operations

id1 = id + 100; % We are trying to get every image close to white.

% Embed function for this is

% id1 = imadd(myimage,100);

figure(Name="Section 3");

imshow(uint8(id1))

Decrease the Intensity :

S = r - C  where C is constant and r is pixel value of the image and S is the output image.

%% Section 4

% Decrease the intensity of the pixel

id2 = id - 100; % We are trying to get every image close to black.

% Embed function for this is

% id2 = imsubtract(myimage,100);

figure(Name="Section 4");

imshow(uint8(id2))

Multiply Image Pixels :

%% Section 5

% Multiply image pixels

id3 = immultiply(myimage,0.5);

figure(Name="Section 5");

imshow(uint8(id3))

Divide Image Pixels :

%% Section 6

% Divide image pixels

id4 = imdivide(myimage,0.5);

figure(Name="Section 6");

imshow(uint8(id4))

Complement of Image:

%% Section 7

% Complement

% In the complement of a binary image, zeros become ones and ones become zeros.

% Black and white are reversed.

% In the complement of a grayscale or color image, each pixel value is subtracted

% from the maximum pixel value supported by the class (or 1.0 for double-precision images).

% The difference is used as the pixel value in the output image. In the output image,

% dark areas become lighter and light areas become darker. For color images,

% reds become cyan, greens become magenta, blues become yellow, and vice versa.

% --

% Get the maximum value of the image

max_value_image= max(myimage(:));

% Max value for 8 bits monochrome image is 255 (0-255)

id5 = abs(id - 255);

% Embed function for this is

% id5 = imcomplement(myimage);

figure(Name="Section 7");

imshow(uint8(id5))

Logarithm Operator : Reducing contrast of brighter regions

S = C * log(1 + r) where C is constant and r is pixel value of the image and S is the output image.

%% Section 8

% Logarithm Operator : Reducing contrast of brighter regions

id_double = im2double(myimage);

% Equation

% S = C * log(1 + r) where C is constant and r is pixel value of the image and S is the output image.

% Assign image height and width to r and c variables

[r,c] = size(id_double);

% Define 5 Constant value for C

factor = [1 6 12 25 100];

% Plot the original image to compare

subplot (1,6,1); imshow(id_double); title("Original Image");

% Run image processing equation for 5 Constant value

for im_num = 1:5 % for each constant value , means for each image

for i = 1:r

for j = 1:c

S(i,j) = factor(im_num) * log(1 + id_double(i,j)); % C = factor(im_num) , r = id(i,j)

end

end

%

subplot (1,6,im_num+1); imshow(S); title(sprintf("Constant Value C = %d", factor(im_num)));

end

Exponential/Raise to Power Operator : Enhancing contrast of brighter regions

Power function: S = C * ( r ^ Y  ) where 0<=s <=1 , 0<=r <=1

%% Section 9

% Power Function : Enhancing contrast of brighter regions

id_double = im2double(myimage);

% Equation

% S = C * ( r ^ Y ) where C is constant and r is pixel value of the image and S is the output image.

% Y = 2 in this example.

% Assign image height and width to r and c variables

[r,c] = size(id_double);

% Define 5 Constant value for C

factor = [1 6 12 25 100];

% Plot the original image to compare

subplot (1,6,1); imshow(id_double); title("Original Image");

% Run image processing equation for 5 Constant value

for im_num = 1:5 % for each constant value , means for each image

for i = 1:r

for j = 1:c

S(i,j) = factor(im_num) * id_double(i,j) ^ 2; % C = factor(im_num) , r = id(i,j)

end

end

%

subplot (1,6,im_num+1); imshow(S); title(sprintf("Constant Value C = %d", factor(im_num)));

end

Histogram Equalization: General method of modifying intensity distribution

Spatial filters : To change the grey level of a given pixel we need only know the value of the grey levels in small neighbourhood of pixels around the given pixel.

Point Operations: A pixels grey value is changed without any knowled of its surrounds.

Contrast Stretching : Spreading out gray level distribution